1.   Prepare a USB disk（USB drive）

2. Right click and select format... Format USBdisk to FAT32 format,and change the

Volume label to WINPE ,click start to finish formatting

- Copy the files to USB disk as below, and it must be in the root directory of USB disk.

(The specific file is subject to the file you open,and the following just a sample)

4. Restart the computer, press F7 repeatedly on  the boot logo interface, and select

USB boot in the boot options

5.If the BIOS flashes automatically, wait till it finishes. If not, type fs3:(you may type fs1:-fs4:,depends on where the USB drive is ,do not forget the colon) , then press enter

6.Type dir or ls , press enter so that you can locate the files

7.Type flash, press tab,then press enter.

8.Start flash, If there is a similar reminder, enter exit.